Mini Project 2 — 4-DOF Robotic Arm Controlled via MATLAB GUI and Arduino

Students: Maria Luiza Cabral, Philip Pham, Gerald Springstead and Jordan Nance.

Professor: Pallavi Singh

Class: EEL3013.82833

1) Project Description

This project presents a 4-degree-of-freedom (4-DOF) robotic arm designed for basic pick-and-place operations. The system integrates a MATLAB GUI and an Arduino Mega 2560 for motion control and real-time user interaction. The GUI allows users to control servo angles and base rotation. The project uses 3D-printed components adapted from the Easy Robot Arm design by mobiobi on Thingiverse.

2) Components Used

2.1) Hardware:

- Arduino Mega 2560

- 4 × SG90 servo motors (for arm joints and gripper)

- 1 × 28BYJ-48 stepper motor (for base rotation)

- 1 × ULN2003 driver board (to control the stepper motor)

- Power supply (5 V DC)

- 3D-printed parts: gripper, base, arm links, and turntable

- M3 screws, nuts, and small elastic bands (for joint assembly and vibration damping)

2.1) Software:

- MATLAB

- Arduino IDE

- Easy Robot Arm CAD model by mobiobi (Thing:4782837)

3) Working Principle

The user operates the robotic arm through sliders and buttons in the MATLAB GUI. Each slider changes the position of one SG90 servo motor that controls the arm joints and gripper. The Arduino Mega 2560 receives the commands from MATLAB and moves the servos to the selected angles. The 28BYJ-48 stepper motor rotates the base in both directions using dedicated GUI buttons. A “Home” button allows the user to return all servos to their default centered position for calibration.

4) Circuit Diagram

5) MATLAB Code

The following code section shows part of the MATLAB App Designer program used to control the robotic arm. This version demonstrates the main logic for connecting to the Arduino and controlling servo motors through the GUI. The complete MATLAB file RobotArmControl.m (LINK IS HERE) is available at the link.

%% Connect Button Callback

% Establishes connection between MATLAB and Arduino Mega

app.a = arduino('COM5','Mega2560','Libraries',{'Servo'});

% Initialize servos

app.s = cell(1, numel(app.servoPins));

for i = 1:numel(app.servoPins)

app.s{i} = servo(app.a, app.servoPins{i});

writePosition(app.s{i}, 0.5); % 90 degrees (center)

end

6) System Picture

7) Video Demonstration

Video Demonstration

8) Expected Results

The robotic arm was able to move through its joints, but the SG90 servo motors did not have enough torque to lift the arm completely. The MATLAB GUI and Arduino communication worked correctly, and all commands were sent and received as expected. Future improvements include using stronger servos and a stable external power supply to achieve smoother and more reliable motion.

9) Conclusion

The project successfully demonstrated the integration of MATLAB and Arduino for controlling a 4-DOF robotic arm. Although the servos had limited power, the system worked as intended for testing and learning purposes. This project showed the importance of matching component strength with mechanical requirements in real robotic designs.